Product Requirements Rev n.n

Summary: This document outlines the requirements for the Aigen ground vehicle platform. The aerial platform requirements are detailed in a separate document.

- Product Name

  - Ground Robotics Platform

- Product Function

  - Provide herbicide free weed control

  - Generate reports

  - House and protect aerial robot

  - Launch aerial robot

- Who is it for?

  - Agriculture professionals, farmers, and technicians at large enterprise farms

- Speed

  - 4 acres covered per day with no scouting (~1.5 mph).

  - 12 acres per day with scouting

  - Min 1.5 mph

- Dimensions

  - Must fit in box with less than 165 inches in combined length and girth (UPS limit)

  - Can turn within 5 feet of crop end rows

  - Adjustable width from 20” to 30”. Clears 10” furrows and 14” sugarbeets. (Note the sugarbeets do conform out of the way of the robot)

Figure 1: Sugarbeets Example

- Weight

  - Max 110 lbs without packaging

  - based on 150 lbs UPS shipping limit

- Power Source

  - Solar for daytime operation

  - Battery packs for nighttime operation and when solar energy doesn’t provide enough power for operation

- User Interface

  - Cell phone application

  - Ability to outline area

  - Ability to see location of all robots

  - Ability to control robots manually from location

  - Ability to schedule inspections or actions at the farm

  - Reports generated on ground robot show crop analytics and recommendations

Figure 2: Example Survey Screen

- Weed Control Mechanism:

  - Kill 5,000 weeds per acre and 4 acres a day, 0.23 weeds per second. Or one every 4 seconds.

  - Herbicide free

- Control

  - Autonomous with customer manual override

- Communications

  - Close range high data transfer rate to WiFi networks at farms

  - Mesh communications between robots

  - Cellular networking ability for farm connectivity

  - GPS for location tracking

- Price

  - Maximum cost to manufacture: $2,500

- Environmental Conditions

  - Needs to climb slopes up to 35 degrees

  - Must operate in temperature ranges from -20 to 45 degrees Celsius

  - Will operate in conditions at farms. Can be kept outdoors year round through, rain, snow, direct sun exposure, and muddy ground conditions through irrigation lanes

  - Will be shipped to customers via UPS or FedEx

- Lifecycle

  - Designed to last 8 years with no maintenance.

  - 8 year warranty period